# Auckland Council

# Rain Monitoring System Documentation

INTERNAL DOCUMENT

Generated: 26 December 2025

Version 1.0 - DRAFT

# Table of Contents

1. Introduction
2. System Architecture
3. Rain Gauge Monitoring
4. Rain Radar Monitoring
5. ARI Calculation Methodology
6. Alarm System
7. Technical Reference
8. Appendices

# 1. Introduction

## 1.1 Purpose

This document provides comprehensive documentation of Auckland Council's rain monitoring system, which consists of two complementary data sources: rain gauges providing point measurements and rain radar providing spatial coverage across stormwater catchments.

## 1.2 Scope

This documentation covers:

• Rain gauge network configuration and alarm setup
• Rain radar (QPE) data collection and processing
• ARI (Annual Recurrence Interval) calculation methodology
• Alarm trigger logic and historical analysis
• Technical reference for data access via Moata API

## 1.3 Definitions and Acronyms

| Term | Definition |
| --- | --- |
| ARI | Annual Recurrence Interval - the average time between events of a given magnitude |
| QPE | Quantitative Precipitation Estimate - radar-derived rainfall estimates |
| TP108 | Technical Publication 108 - NIWA methodology for rainfall frequency analysis |
| Moata | Auckland Council's asset management and monitoring platform |
| Trace | A time series data stream from a sensor or calculated value |
| Threshold | A configured value that triggers an alarm when exceeded |
| Catchment | A geographic area that drains to a common outlet |

# 2. System Architecture

## 2.1 Overview

The rain monitoring system integrates data from multiple sources through the Moata platform, which provides unified access to sensor data, alarm configuration, and historical records.

## 2.2 Data Flow

Data flows through the system as follows:

1. Data Collection: Rain gauges and radar systems collect precipitation data
2. Data Ingestion: Raw data is ingested into the Moata platform
3. Processing: Data is processed, including ARI calculations for radar data
4. Alarm Evaluation: Processed values are compared against configured thresholds
5. Notification: Exceedances trigger alarms and notifications

## 2.3 Data Sources Summary

| Source | Coverage | Resolution | Primary Use |
| --- | --- | --- | --- |
| Rain Gauges | 76 active stations | 5-minute intervals | Point measurements, ARI alarms |
| Rain Radar | 233 catchments 25,298 pixels | 1-minute intervals | Spatial coverage, area exceedance |

# 3. Rain Gauge Monitoring

## 3.1 Network Coverage

The rain gauge network consists of physical sensors deployed across the Auckland region. Not all gauges in the system are actively monitored for alarms.

| Category | Count |
| --- | --- |
| Total gauges in dataset | 264 |
| Excluded: Non-Auckland (by keyword) | 67 |
| Excluded: No physical sensor (forecast/nowcast only) | 44 |
| Excluded: No recent telemetered data | 77 |
| Active Auckland rain gauges | 76 |

## 3.2 Filtering Criteria

Gauges are filtered based on the following criteria to identify active Auckland stations:

Step 1 - Auckland Location: Gauge name must contain Auckland-related keywords (ACC, Auckland, NSCC, Metrowater, etc.) and must not contain exclusion keywords (WSL, Watercare, WDC, etc.)

Step 2 - Physical Sensor: Gauge must have a physical rainfall sensor trace (not just forecast or nowcast data)

Step 3 - Recent Data: Gauge must have telemetered data within the last 3 months

## 3.3 Trace Types

Each rain gauge can have multiple data traces. The following trace types are present in the network:

| Trace Type | Description |
| --- | --- |
| Rainfall (primary) | Raw rainfall measurement from sensor |
| Rainfall 60 min window sum | Rolling 60-minute rainfall total |
| Max TP108 ARI | Maximum ARI value across all durations |
| Rain Nowcast | Short-term rainfall prediction |
| Rain Forecast | Longer-term rainfall forecast |

## 3.4 Alarm Configuration

Alarms are configured on the rain gauge network to detect significant rainfall events and data quality issues.

| Alarm Type | Count | Description |
| --- | --- | --- |
| Overflow Monitoring | 694 | Threshold-based alarms on rainfall/ARI values |
| Data Recency | 76 | Alerts when data stops flowing from a gauge |

ARI Threshold: The standard threshold for ARI-based alarms is 5 years. This means an alarm is triggered when the calculated ARI exceeds 5 years, indicating rainfall intensity that would be expected to occur on average once every 5 years.

# 4. Rain Radar Monitoring

## 4.1 Spatial Coverage

Rain radar provides spatial coverage of precipitation across the Auckland region. Data is organized by stormwater catchments, with each catchment mapped to multiple radar pixels.

| Metric | Value |
| --- | --- |
| Stormwater catchments | 233 |
| Total radar pixels | 25,298 |
| Average pixels per catchment | ~109 |
| Pixels with TP108 coefficients | 19,356 |

## 4.2 Data Characteristics

| Parameter | Value |
| --- | --- |
| Data Type | QPE (Quantitative Precipitation Estimate) |
| Temporal Resolution | 1 minute |
| Spatial Resolution | Radar pixel (~1km²) |
| TraceSet Collection ID | 1 |
| TraceSet ID | 3 |
| API Limit - Pixels per request | 150 (recommended: 50) |
| API Limit - Time range | 24 hours per request |

## 4.3 Pixel-to-Catchment Mapping

Each stormwater catchment is associated with a set of radar pixels that intersect its geographic boundary. This mapping is:

• Static - pixel indices do not change over time
• Determined by geometry intersection via the Moata API
• Cached locally to avoid repeated API calls
• Stored in JSON and pickle formats for efficient reuse

# 5. ARI Calculation Methodology

## 5.1 TP108 Background

The TP108 methodology, developed by NIWA (National Institute of Water and Atmospheric Research), provides a standardized approach for calculating Annual Recurrence Intervals (ARI) from rainfall data. The methodology accounts for regional variations in rainfall patterns through pixel-specific coefficients.

## 5.2 Formula

The ARI is calculated using the following formula:

ARI = exp(m × D + b)

Where:
• ARI = Annual Recurrence Interval (years)
• D = Rainfall depth (mm) accumulated over the duration
• m = Slope coefficient (specific to pixel and duration)
• b = Intercept coefficient (specific to pixel and duration)

## 5.3 Durations

ARI is calculated for eight standard durations. Shorter durations capture intense bursts while longer durations capture sustained rainfall events.

| Duration | Minutes | Typical Depth for ARI=5* |
| --- | --- | --- |
| 10 minutes | 10 | ~13 mm |
| 20 minutes | 20 | ~21 mm |
| 30 minutes | 30 | ~26 mm |
| 1 hour | 60 | ~36 mm |
| 2 hours | 120 | ~49 mm |
| 6 hours | 360 | ~75 mm |
| 12 hours | 720 | ~97 mm |
| 24 hours | 1440 | ~119 mm |

*Note: Depths shown are approximate and vary by pixel location. Values shown are for a representative Auckland pixel.

## 5.4 Catchment-Level Alarming

For stormwater catchments, the alarm system monitors the proportion of the catchment area where ARI exceeds the threshold. This is calculated by:

1. Computing ARI for each pixel within the catchment
2. Identifying pixels where ARI exceeds the threshold (5 years)
3. Calculating the proportion of total catchment pixels exceeding
4. Triggering alarm if proportion exceeds configured threshold

# 6. Alarm System

## 6.1 Alarm Types

Overflow Monitoring
Triggered when a measured or calculated value exceeds a configured threshold. For rain gauges, this typically monitors the "Max TP108 ARI" trace with a threshold of 5 years.

Data Recency
Triggered when data from a sensor has not been received within a configured time period. This helps identify sensor failures or communication issues.

## 6.2 Historical Alarm Analysis

Analysis of historical alarm events from May to December 2025 shows the following patterns:

| Month | Alarm Events |
| --- | --- |
| May 2025 | 5 |
| June 2025 | 8 |
| July 2025 | 1 |
| August 2025 | 2 |
| September 2025 | 1 |
| November 2025 | 11 |
| December 2025 | 10 |
| Total | 38 |

Note: Higher alarm counts in November and December 2025 correspond to increased rainfall activity during the spring/summer storm season.

# 7. Technical Reference

## 7.1 Moata API Endpoints

| Endpoint | Purpose |
| --- | --- |
| GET /v1/projects/{projectId}/assets | Get assets (gauges, catchments) with optional geometry |
| GET /v1/assets/traces | Get traces for one or more assets |
| GET /v1/traces/{traceId}/data/utc | Get time series data for a trace |
| GET /v1/traces/{traceId}/thresholds | Get alarm thresholds for a trace |
| GET /v1/alarms/overflow-detailed-info-by-trace | Get alarm configuration for a trace |
| GET /v1/trace-set-collections/{id}/trace-sets/data | Get radar data for pixels |
| GET /v1/trace-set-collections/{id}/pixel-mappings/intersects-geometry | Get pixels for a geometry |

## 7.2 Key Configuration Values

| Parameter | Value |
| --- | --- |
| Project ID | 594 |
| Rain Gauge Asset Type ID | 25 |
| Stormwater Catchment Asset Type ID | 3541 |
| TraceSet Collection ID (Radar) | 1 |
| TraceSet ID (QPE) | 3 |
| ARI Threshold | 5 years |
| Coordinate System (SRID) | 4326 (WGS84) |

## 7.3 Data Pipeline Scripts

| Script | Purpose |
| --- | --- |
| retrieve_rain_gauges.py | Collect rain gauge data, traces, and alarm configs |
| retrieve_rain_radar.py | Collect radar data for stormwater catchments |
| analyze_rain_gauges.py | Analyze and filter rain gauge data, generate reports |

# 8. Appendices

## Appendix A: Output File Structure

outputs/
├── rain_gauges/
│   ├── raw/
│   │   └── rain_gauges_traces_alarms.json
│   └── analyze/
│       ├── alarm_summary.csv
│       ├── alarm_summary_full.csv
│       ├── all_traces.csv
│       ├── analysis_report.txt
│       └── dashboard.html
└── rain_radar/
    └── raw/
        ├── catchments/
        │   └── stormwater_catchments.csv
        ├── pixel_mappings/
        │   ├── catchment_pixel_mapping.json
        │   └── catchment_pixel_mapping.pkl
        ├── radar_data/
        │   └── {catchment_id}_{name}.csv
        └── collection_summary.json

## Appendix B: Glossary of Trace Descriptions

The following trace descriptions are found in the rain gauge network. See the generated alarm_summary_full.csv for the complete list with associated alarm configurations.

## Appendix C: Catchment Inventory

A complete list of 233 stormwater catchments with pixel counts is available in the collection_summary.json output file.